# SMART VOTING SYSTEM USING ARDUINO UNO

## Abstract

The Smart Voting System using Arduino Uno is an embedded system project designed to provide a secure, simple, and efficient electronic voting mechanism. The system allows users to cast votes through push buttons connected to an Arduino Uno board. The votes are counted digitally and displayed on an LCD screen in real time. The project demonstrates the practical application of microcontrollers in automation, voting systems, and embedded electronics.

The system ensures quick vote counting, reduces manual errors, and provides immediate result display. It also includes a reset mechanism for conducting multiple voting sessions. This project is suitable for educational institutions, organizational polling, and small-scale election systems.

# 1. Introduction

Voting is an important process in democratic systems and organizational decision-making. Traditional voting methods often require manual counting, which can consume time and may lead to errors. Electronic voting systems solve these problems by automating vote collection and result generation.

The Smart Voting System using Arduino Uno is developed to simplify the voting process using embedded technology. The system uses push buttons for vote casting, an LCD module for displaying candidate names and results, and LEDs for status indication. The Arduino Uno acts as the central controller that processes all voting operations.

This project focuses on:

- Real-time vote counting

- Easy user interaction

- Accurate result generation

- Compact and low-cost implementation

- Secure and transparent operation

# 2. Objectives

The major objectives of the project are:

- To design a smart electronic voting system using Arduino Uno.

- To provide a simple push-button voting interface.

- To display candidate information and voting confirmation on an LCD.

- To count votes automatically and accurately.

- To display final voting results instantly.

- To reduce manual counting errors.

- To create a reusable and scalable voting solution.

# 3. Project Overview

The Smart Voting System allows users to cast votes for candidates using push buttons. Each button represents a candidate. Once a voter presses a button, the corresponding vote count increases and the LCD displays confirmation.

At the end of voting, the result button is pressed to display the winner and total votes. The system can then be reset for another voting session.

### Main Features

- Easy voting using push buttons

- Real-time LCD display

- Instant vote counting

- Winner declaration

- Secure and tamper-resistant design

- Simple and reliable operation

# 4. Hardware Components Used

| Component | Description |
| --- | --- |
| Arduino Uno | Main microcontroller board used for processing |
| LCD Display (16x2 I2C) | Displays candidate names, messages, and results |
| Push Buttons | Used for voting and result display |
| LEDs | Indicate voting and result status |
| Breadboard | Used for circuit connections |
| Jumper Wires | Provide electrical connections |
| USB Cable | Powers and programs Arduino Uno |

# 5. Description of Components

## 5.1 Arduino Uno

Arduino Uno is the main controller of the system. It receives input signals from push buttons, processes votes, updates counters, and controls the LCD and LEDs.

### Specifications

- ATmega328P microcontroller

- Operating voltage: 5V

- Digital I/O pins: 14

- Analog input pins: 6

- Clock speed: 16 MHz

## 5.2 LCD Display (16x2 I2C)

The LCD module displays:

- Candidate names

- Vote confirmation messages

- Final voting results

The I2C interface reduces wiring complexity by using only two communication lines.

## 5.3 Push Buttons

Five push buttons are used:

- Button A → Candidate A

- Button B → Candidate B

- Button C → Candidate C

- Button D → Candidate D

- Result Button → Displays winner and vote count

## 5.4 LEDs

Two LEDs are used:

- LED 12 → Blinks during vote casting

- LED 13 → Indicates result display

# 6. Working Principle

The working of the Smart Voting System is divided into several stages.

## Step 1: System Initialization

When the Arduino is powered ON:

- LCD initializes

- Pins and variables are configured

- Candidate names are displayed

## Step 2: Vote Casting

The voter presses one of the candidate buttons:

- A, B, C, or D

- Arduino detects button input

- Corresponding vote counter increases

- LED 12 blinks

- LCD displays confirmation message

## Step 3: Vote Counting

Votes are stored in variables inside the Arduino program.

Example:

- vote1 → Candidate A

- vote2 → Candidate B

- vote3 → Candidate C

- vote4 → Candidate D

## Step 4: Result Display

When the RESULT button is pressed:

- Arduino compares all vote counts

- Determines the winner

- Displays winner name and vote count on LCD

- LED 13 blinks

## Step 5: System Reset

After displaying results:

- Vote counters are reset

- LCD displays candidate names again

- System becomes ready for next round

# 7. System Flow

The operational flow of the project is:

- Start the system

- Initialize Arduino and LCD

- Display candidate names

- Wait for button press

- Increment selected candidate vote count

- Show vote confirmation

- Return to candidate selection screen

- On pressing RESULT button:

  - Calculate winner

  - Display results

- Reset vote counters

- Restart voting process

# 8. Circuit Connections

## LCD Connections

| LCD Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

## Push Button Connections

| Button | Arduino Pin |
| --- | --- |
| Candidate A | Pin 3 |
| Candidate B | Pin 4 |
| Candidate C | Pin 5 |
| Candidate D | Pin 6 |
| Result Button | Pin 7 |

## LED Connections

| LED | Arduino Pin |
| --- | --- |
| Vote LED | Pin 12 |
| Result LED | Pin 13 |

# 9. Arduino Program Explanation

The Arduino program controls the entire voting system.

## Important Functions

### setup()

The setup() function:

- Initializes LCD

- Configures input and output pins

- Displays startup messages

### loop()

The loop() function continuously:

- Checks button presses

- Increments votes

- Displays confirmations

- Shows results when requested

- Resets the system

## Variables Used

| Variable | Purpose |
| --- | --- |
| vote1 | Stores votes for Candidate A |
| vote2 | Stores votes for Candidate B |
| vote3 | Stores votes for Candidate C |
| vote4 | Stores votes for Candidate D |

# 10. Sample Arduino Code

#include <Wire.h>
#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

#define v1 3
#define v2 4
#define v3 5
#define v4 6
#define v5 7

int vote1 = 0, vote2 = 0, vote3 = 0, vote4 = 0;

void setup() {
    pinMode(v1, INPUT_PULLUP);
    pinMode(v2, INPUT_PULLUP);
    pinMode(v3, INPUT_PULLUP);
    pinMode(v4, INPUT_PULLUP);
    pinMode(v5, INPUT_PULLUP);

    pinMode(12, OUTPUT);
    pinMode(13, OUTPUT);

    lcd.init();
    lcd.backlight();

    lcd.setCursor(0,0);
    lcd.print("Smart Voting");
    delay(2000);
    lcd.clear();
}

void loop() {

    if(digitalRead(v1) == LOW) {
        vote1++;
        lcd.clear();
        lcd.print("Vote for A");
        digitalWrite(12, HIGH);
        delay(1000);
        digitalWrite(12, LOW);
    }

    if(digitalRead(v2) == LOW) {
        vote2++;
        lcd.clear();
        lcd.print("Vote for B");
        digitalWrite(12, HIGH);
        delay(1000);
        digitalWrite(12, LOW);
    }

    if(digitalRead(v3) == LOW) {
        vote3++;
        lcd.clear();
        lcd.print("Vote for C");
        digitalWrite(12, HIGH);
        delay(1000);
        digitalWrite(12, LOW);
    }

    if(digitalRead(v4) == LOW) {
        vote4++;
        lcd.clear();
        lcd.print("Vote for D");
        digitalWrite(12, HIGH);
        delay(1000);
        digitalWrite(12, LOW);
    }

    if(digitalRead(v5) == LOW) {
        lcd.clear();

        if(vote1 > vote2 && vote1 > vote3 && vote1 > vote4)
            lcd.print("A Wins");
        else if(vote2 > vote1 && vote2 > vote3 && vote2 > vote4)
            lcd.print("B Wins");
        else if(vote3 > vote1 && vote3 > vote2 && vote3 > vote4)
            lcd.print("C Wins");
        else if(vote4 > vote1 && vote4 > vote2 && vote4 > vote3)
            lcd.print("D Wins");
        else
            lcd.print("Tie");

        digitalWrite(13, HIGH);
        delay(3000);
        digitalWrite(13, LOW);

        vote1 = vote2 = vote3 = vote4 = 0;
    }
}

# 11. Advantages

- Fast and accurate voting system

- Reduces manual errors

- Easy to operate

- Low-cost implementation

- Real-time result display

- Compact and portable design

- Reusable for multiple voting sessions

# 12. Limitations

- Suitable mainly for small-scale voting

- No biometric authentication

- Limited number of candidates

- Requires stable power supply

- No remote voting capability

# 13. Applications

The Smart Voting System can be used in:

- College and university elections

- Organizational polling

- Event voting systems

- Opinion polls and surveys

- Classroom voting activities

- Small-scale secure elections

# 14. Future Enhancements

The project can be improved by adding:

- Fingerprint authentication

- RFID voter identification

- Internet-based result monitoring

- Touchscreen interface

- Cloud database integration

- Wireless voting modules

- Mobile application support

# 15. Conclusion

The Smart Voting System using Arduino Uno successfully demonstrates an efficient electronic voting mechanism using embedded systems technology. The project provides a secure, accurate, and user-friendly voting solution with real-time vote counting and instant result display.

By using Arduino Uno, push buttons, LEDs, and LCD display, the system automates the voting process while reducing human effort and counting errors. The project is economical, easy to implement, and suitable for educational and organizational applications.

This project also serves as an excellent learning platform for students interested in embedded systems, automation, and Arduino programming.

# 16. References

- Arduino Uno Official Documentation

- Arduino IDE Programming Guide

- LiquidCrystal_I2C Library Documentation

- Embedded Systems Fundamentals

- Microcontroller-Based System Design Books